## CS422 Homework 5

1. True or False

(a) True
(b) True
(c) False
(d) False
(e) False

### 2. Slotted ALOHA vs Pure ALOHA vs CSMA/CD

#### (a) Why is Slotted ALOHA more efficient than Pure ALOHA?

Slotted ALOHA cuts the vulnerable period in half, pure ALOHA vulnerable period = 2T , slotted ALOHA vulnerable period = T, (T = frame transmission time), this reduces collisions and increases maximum throughput.

#### (b) Why is CSMA/CD more efficient than Slotted ALOHA? Why can’t CSMA guarantee no collisions?

Why more efficient: CSMA/CD listens to the channel before transmitting and aborts early if it detects a collision. This reduces wasted time compared to blindly transmitting like ALOHA.

Why collisions can still happen:Propagation delay. Two nodes may sense the channel as idle at the same time and transmit simultaneously before the signals reach each other.

### 3. Choosing the best multiple-access protocol

#### (a) 10 servers, always sending (infinite traffic), best choice: Token passing (or TDMA)

Under heavy load, deterministic channel access (token passing or TDMA) prevents collisions and guarantees fair usage.

#### (b) 10 hosts with bursty Internet traffic, best choice: CSMA/CD

Contention-based and decentralized. Efficient for light or bursty traffic. No wasted slots like in TDMA.

### 4. Subnet routing

#### (a) Communication E - F

- Will Router1 receive the frame? No

- Source IP: 192.168.3.001

- Destination IP: 192.168.3.003

- Source MAC: MAC of E

- Destination MAC: MAC of F

Because E and F are on the same LAN, the frame stays local.

#### (b) Communication E - B

- Will Router1 receive the frame? Yes, when Router2 forwards it.

- First frame created by E:

  - Source IP: 192.168.3.001

  - Destination IP: 192.168.1.003

  - Source MAC: MAC of E

  - Destination MAC: MAC of Router2 (default gateway)

- Minimum number of link-layer frames: 3

  - E - Router2

  - Router2 - Router1

  - Router1 - B

#### (c) ARP behavior between A and B on same LAN

- Does Router1 receive A’s ARP request? Yes, because ARP is broadcast.

- Will Router1 forward the ARP to subnet 2? No, routers do not forward link-layer broadcasts.

- Will B send an ARP request to learn A’s MAC? No, B simply replies to A’s ARP request.

- Will Router1 receive the data frame B → A? No, this is unicast and the switch sends it only to A.

### 5. Switch learning

Frames sent in this order: C – I, I – C, F – C

Initial state: all switch tables empty.

#### (a) After frame C - I

- S1: C - port 4

- S4: C - port 1

- S2: C - port 1

- S3: C - port 1

All switches learned only C; destination was unknown so the frame was flooded.

#### (b) After frame I - C

- S3: I - port 4

- S4: I - port 3

- S1: I - port 1

- S2: unchanged

#### (c) After frame F - C

- S2: F - port 4

- S4: F - port 2

- S1: F - port 1

- S3: unchanged

#### (d) Which nodes physically receive the frame F - C?

Path taken: F - S2 - S4 - S1 - C

Nodes that physically receive the frame: S2, S4, S1, C

Nodes that do NOT receive it: A, B, D, E, G, H, I