CS 25000 Lab 12 – upload by the end of your lab session. Work in a team of two or three.

- By convention, a cache is named according to the amount of data it contains (i.e., a 4 KiB cache can hold 4 KiB of data); however, caches also require SRAM to store metadata such as tags and valid bits.
    Calculate the total number of bits required to implement a byte-addressable, direct-mapped, write-through 32 KiB cache with two-word blocks, 64-bit words, and 64-bit addresses.

Number of Blocks/Sets: 32,768 bytes / 16 bytes/block = 2,048 sets. 2.

Address Bit Breakdown (64 bits):

- Offset Bits: log2(16) = 4 bits.

- Index Bits (Set): log2(2048) = 11 bits.

- Tag Bits: 64 - 11 - 4 = 49 bits. 3.

Bits per Set/Row:

- Data Bits: 16 bytes * 8 bits/byte = 128 bits.

- Metadata Bits: 49 Tag bits + 1 Valid bit (Write-through does not require a Dirty Bit).

- Total per row: 128 + 49 + 1 = 178 bits. 4.

Total Bits Required:    178 bits/set * 2,048 sets = 364,544 bits.

Answer: 364,544 bits.

- Calculate the total number of bits required to implement a 64 KiB cache with 16-word blocks and all other parameters the same as the cache of Question 1. How much bigger is this cache than the 32 KiB cache described in Question 1? (Notice that, by changing the block size, we doubled the amount of data without doubling the total size of the cache.)

Number of Blocks/Sets: 65,536 bytes / 128 bytes/block = 512 sets.

Address Bit Breakdown (64 bits):

- Offset Bits: log2(128) = 7 bits.

- Index Bits (Set): log2(512) = 9 bits.

- Tag Bits: 64 - 9 - 7 = 48 bits.

Bits per Set/Row:

- Data Bits: 128 bytes * 8 bits/byte = 1,024 bits.

- Metadata Bits: 48 Tag bits + 1 Valid bit.

- Total per row: 1,024 + 48 + 1 = 1,073 bits.

Total Bits Required:

1,073 bits/set * 512 sets = 549,376 bits.

Comparison:

- Cache 1 (32 KiB): 364,544 bits.

- Cache 2 (64 KiB): 549,376 bits.

- Difference: 549,376 - 364,544 = 184,832 bits.

Answer: 549,376 bits. It is 184,832 bits larger (approximately 1.51 times).

- Explain why the 64 KiB cache of Question 2, despite its larger data size, might provide slower performance than the 32 KiB cache of Question 1. Specifically, compare the hit times and the effects of compulsory misses and conflict misses between the two cache designs.

The 64 KiB cache (Question 2) might provide slower performance than the 32 KiB cache (Question 1) for the following reasons:

Hit Time: Generally, larger caches have a longer hit time due to physical delays (longer signal paths) and the complexity of the decoding circuit. Additionally, the larger block size (128 bytes) in the 64 KiB cache can increase the time required to read or select the data.

Conflict Misses: The 64 KiB cache, despite having double the data capacity, has significantly fewer sets (512) than the 32 KiB cache (2,048). A lower number of sets increases the probability that different main memory blocks will compete and map to the same index, leading to more conflict misses. 3.

Compulsory Misses: The 64 KiB cache has larger blocks (128 bytes). This better exploits spatial locality, so the number of compulsory misses (cold misses) tends to be lower compared to the 32 KiB cache (16-byte blocks).

- For a direct-mapped cache design with a 64-bit address, the Tag bits are in positions 63 through 10, the set number bits in positions 9 through 5, and the block byte address bits are in positions 4 through 0. What is the ratio between total bits required for such a cache implementation over the data storage bits?

Block Size: $2^5 = 32$ bytes.

Data Bits per Block: $32 \text{ bytes} \times 8 \text{ bits/byte} = 256$ bits.

Total Bits per Block: 256 (Data) + 54 (Tag) + 1 (Valid bit) = 311 bits.

Total/Data Ratio:    311 bits / 256 bits $\approx 1.21$

Answer: The ratio between the total required bits and the data storage bits is $\mathbf{311/256}$ (or $\approx 1.21$).

- Media applications that play audio or video files are part of a class of workloads called "streaming" workloads i.e., they bring in large amounts of data but do not reuse much of it. Consider a video streaming workload that accesses a 512 KiB working set sequentially with the following address stream:

0, 1, 2, 3, 4, 5, 6, 7, 8, 9 …

Assume a 64 KiB direct-mapped cache with a 32-byte block. Assume the address stream is  word addresses of LEGv8 words, i.e., 32-bit words.
     What is the miss rate for the address stream above? How is this miss rate sensitive to the size of the cache or the working set? How would you categorize the misses this workload is experiencing, based on the 3C model?

Word Size: 32 bits = 4 bytes.

Words per Block: 32 bytes / 4 bytes/word = 8 words/block.

Miss Rate: The first access to a block misses, bringing in 8 words. The next 7 accesses are hits.

- Miss Rate = 1 miss / 8 accesses = 1/8.

Answer: The miss rate is $\mathbf{12.5\%}$ (or 1/8).

Sensitivity:

- The miss rate is **not very sensitive** to the cache size or the working set size.

- The miss rate is **highly sensitive** to the block size.

Miss Category:

- Based on the 3C model, these misses are predominantly **Compulsory Misses** (Cold Misses).

- Re-compute the miss rate for Question 5 when the cache block size is 16 bytes, 64 bytes, and 128 bytes. What kind of locality is this workload exploiting?

Words per Block = Block Size / 4 bytes. Miss Rate = 1 / Words per Block.

16-byte block: 16/4 = 4 words/block. Miss Rate = 1/4 = $\mathbf{25\%}$.

64-byte block: 64/4 = 16 words/block. Miss Rate = 1/16 = $\mathbf{6.25\%}$.

128-byte block: 128/4 = 32 words/block. Miss Rate = 1/32 = $\mathbf{3.125\%}$.

Locality Exploited:

- The sequential loading of contiguous data exploits **Spatial Locality**.

- "Prefetching" is a technique that leverages predictable address patterns to speculatively bring in additional cache blocks when a particular cache block is accessed. One example of prefetching is a stream buffer that prefetches sequentially adjacent cache blocks into a separate buffer when a particular cache block is brought in. If the data are found in the prefetch buffer, it is considered as a hit, moved into the cache, and the next cache block is prefetched. Assume a two-entry stream buffer; and, assume that the cache latency is such that a cache block can be loaded before the computation on the previous cache block is completed. What is the miss rate for the address stream of Question 5?

Analysis: - Prefetching hides the latency of sequential misses. Only the very first access to the entire sequence misses. Subsequent blocks are always available in the stream buffer when requested.

Answer:The effective miss rate approaches **0%** for a continuous stream.

- In general, cache access time is proportional to capacity. Assume that main memory accesses take 70 ns and that 36% of all instructions access data memory. The following table shows data for L1 caches attached to each of two processors, P1 and P2.

Assuming that the L1 hit time determines the cycle times for P1 and P2, what are their respective clock rates?

P1:

- Cycle Time: 0.66 ns.

- Clock Rate: $1 / (0.66 \times 10^{-9} \text{ s}) \approx \mathbf{1.515 \text{ GHz}}$.

P2:    - Cycle Time: 0.90 ns.    - Clock Rate: $1 / (0.90 \times 10^{-9} \text{ s}) \approx \mathbf{1.111 \text{ GHz}}$.

- What is the Average Memory Access Time for P1 and P2 of Question 8 in clock cycles and in nanoseconds?

Formula: AMAT = Hit Time + Miss Rate $\times$ Miss Penalty.

Miss Penalty (ns): 70 ns.

P1: (Hit Time: 0.66 ns, Miss Rate: 8.0%)

- AMAT (ns): $0.66 + (0.08 \times 70) = \mathbf{6.26 \text{ ns}}$.    –

AMAT (cycles): $1 + (0.08 \times 70 / 0.66) \approx \mathbf{9.48 \text{ cycles}}$.

P2: (Hit Time: 0.90 ns, Miss Rate: 6.0%)

- AMAT (ns): $0.90 + (0.06 \times 70) = \mathbf{5.10 \text{ ns}}$.

- AMAT (cycles): $1 + (0.06 \times 70 / 0.90) \approx \mathbf{5.67 \text{ cycles}}$.

- Assuming a base CPI of 1.0 without any memory stalls, what is the total CPI for P1 and P2 of Question 8? Which processor is faster?

Formula: Total CPI = Base CPI + (Mem Accesses per Inst) $\times$ Miss Rate $\times$ Miss Penalty (cycles).

- Base CPI: 1.0.

- Mem Accesses per Inst: 1 (Inst Fetch) + 0.36 (Data Access) = 1.36.

P1:

- Total CPI: $1.0 + (1.36 \times 0.08 \times 106.06) \approx \mathbf{12.54}$. 2.

P2:

- Total CPI: $1.0 + (1.36 \times 0.06 \times 77.78) \approx \mathbf{7.35}$.

Faster Processor:

- P1 Time: $12.54 \times 0.66 \text{ ns} \approx 8.28 \text{ ns/inst}$.

- P2 Time: $7.35 \times 0.90 \text{ ns} \approx 6.62 \text{ ns/inst}$.

Answer: **Processor P2 is faster**.