## CS 354 Fall 2025

## Homework 4 [20 pts]

## Due: 10/27/2025 (Mon.), 11:59 PM

1. If the system uses single-level page tables with 8K bytes per page, how many

entries will each page table contain?  Explain how you arrived at the answer.

Page size = 8K bytes = 8192 bytes = 2¹³, offset = 13 bits, and virtual address = 48 bits, page number = 48 − 13 = 35 bits. Number of entries = 2^35.

Each page table contains 2^35 entries.

2. If the system uses single-level page tables with 32K bytes per page, how many

entries will each page table contain?  Explain how you arrived at the answer.

Page size = 32K bytes = 32768 = 2¹⁵, offset = 15 bits, and virtual address = 48 bits, page number = 48 − 15 = 33 bits. Number of entries = 2^33.

Each page table contains 2^33 entries.

3. If the system uses single-level page tables with 8K bytes per page, each entry

in a page table contains a frame number plus a few other items.  How many bits

will be required *just for the frame number*?  Explain how you arrived at the

answer.

Physical memory is indexed by frame number. So each frame holds 8K = 2^13 bytes, so frame number bits = total address bits − offset bits = 48 − 13 = 35 bits.

35 bits are required for the frame number.

4. Imagine the memory references a process makes when using paging.  Most processes make a mixture of references to pages in the text segment, data, bss, and stack segments. However, it is not uncommon to find that half the references occur on the text segment. Explain why.

Every instruction fetch is a memory reference to the text (code) segment. Since the CPU continuously fetches instructions while executing loops and functions, a large fraction of accesses naturally go to text, and spatial and temporal locality cause frequent reuse of the same code pages.

Frequent instruction fetches make the text segment dominate memory references.

The next three questions (5,6,7) use the following assumptions.

Suppose a process running on a paging system performs a machine learning computation that requires the process to access locations of a 1-dimensional array of double floating-point values (i.e., 64-bit floating point values). Also suppose that the array contains 2,000,000 values and the page size is 4K bytes.  Finally, suppose that none of the pages of the array are currently resident in memory.

5. Suppose that the process must access all values of the array sequentially, starting

at location 0.  How many page faults will be caused as the process accesses all

values of the array?  Explain.  (Note: only count page faults for the array

references and assume no other page faults occur.)

Each double = 8 bytes, total array size = 2000000 × 8 = 16000000 bytes, and page size = 4096 bytes, pages needed = ceil(16000000 / 4096) = 3907 pages

3,907 page faults.

6. Modify the previous question slightly.  Suppose that instead of merely accessing

values in the array, the process reads a value, performs a computation, and then

writes an updated value back to the same location.  How many page faults will be

caused as the process accesses all values of the array?  Explain.  (Note: only

count page faults for the array and assume no other page faults occur.)

3,907 page faults, the first access to each page still causes one fault, also subsequent reads/writes to that same page cause no additional faults, and dirty pages are written back later but do not cause more faults.

7. Suppose that instead of sequential access, the process starts at location 0 in the

array, then only accesses every 7th location (i.e., locations 7, 14, 21, 28, ... and

so on).  How many page faults will be caused as the process accesses items in the

array?  Explain.  (As before, only count page faults for the array references and

assume no other page faults occur.)

Step size = 7 elements × 8 bytes = 56 bytes < 4K, each page holds 4096 / 8 = 512 elements, since gcd(7, 512) = 1, the stride pattern eventually touches all offsets and all pages.

3,907 page faults, one per page.

The next two questions (8,9) use the following assumptions.

Consider a paging system that uses two levels of page tables to cover a large address space. The top level page table usually only has some entries filled starting at the bottom of the table and some entries filled at the top of the table, with the middle of the table left empty (i.e., marked to indicate that no second-level page table has been allocated).

Note: we use the term "bottom" to refer to locations 0, 1, and so on (i.e. lowest addresses).

8. Why would the top and bottom be used instead of merely placing all the used entries at the bottom?

Because processes use both low and high virtual addresses, entries appear at both ends.

9. Researchers have studied scheduling algorithms that change a process's priority based on the amount of I/O the process performs, increasing the priority of a process that does lots of I/O and decreasing the priority of a process that does not perform much I/O. Merely counting the times a process invokes I/O systems calls (e.g., *read* and *write* to perform I/O doesn't give a true picture of I/O activity.  What additional information should a memory management unit collect to help assess I/O?  Explain.

Counting I/O system calls alone does not reflect true I/O volume.The MMU or OS should track: Number of bytes transferred per operation, time spent waiting for I/O (latency) and frequency and duration of blocking, finally the number of pages read/written and dirty page counts. So to collect data size, timing, and page-level I/O statistics to measure real I/O intensity.

10. You have been offered a job writing device driver code.  The interviewer remarks that all device drivers are basically the same, and if you've seen one example, you know how much code will be required.  Does the interviewer really understand device drivers?  Explain.

Drivers share basic structure (initialization, interrupt handling, buffering), but real devices differ in interface, timing, synchronization, error handling, and performance constraints. This makes the interviewer is wrong drivers follow general patterns but differ widely in code and complexity.

## Submission

You must submit a directory named hw4 that contains a PDF file (created using dedicated PDF creator or converter):

- questions.pdf, containing your answers.

Go to the directory where hw4 is a subdirectory.

For example, if /homes/alice/cs354/hw4 is your directory structure, go to /homes/alice/cs354.

Type the following command to submit the directory with turnin:

turnin -c cs354 -p hw4 hw4

Be sure the file inside the directory is named exactly questions.pdf.

You can check/list the submitted files using

turnin -c cs354 -p hw4 -v