CS422 Computer Networks (Fall 2025): Homework 3

1. (5 points) TCP waits until it has received three duplicate ACKs before performing a fast retransmit. Why do you think the TCP designers chose not to perform a fast retransmit after the first duplicate ACK for a

segment is received?

TCP designers chose to wait for three duplicate ACKs because a single duplicate ACK might result from packet reordering in the network, where segments arrive out of order but none are lost. A single dup ACK doesn't reliably indicate loss, but three dups suggest that subsequent packets have arrived, implying the missing segment was likely lost. This avoids unnecessary retransmissions that could waste bandwidth.

2. (5 points) Why is a 2-way handshake not sufficient for reliable TCP connection establishment, and why do we need a 3-way handshake instead?

A 2-way handshake is insufficient because it could allow old, duplicated SYN packets from previous connections to erroneously establish a new connection on one side (the server might accept a stale SYN and send SYN-ACK, but the client didn't initiate it). The 3-way handshake ensures synchronization and agreement from both sides: the client's SYN, server's SYN-ACK (acknowledging the client's SYN), and client's ACK (acknowledging the server's SYN). This prevents half-open connections from delayed packets.

3. (6 points) After one party sends the final ACK in response to the peer’s FIN segment during TCP connection termination: Why must this host remain in the waiting state for a certain period before fully closing the connection? If another FIN segment arrives before the timer expires, what will happen, and how does the host handle it?

After sending the final ACK in response to the peer's FIN, the host enters the TIME_WAIT state for 2 * MSL (Maximum Segment Lifetime, typically 2-4 minutes). This wait ensures that if the final ACK is lost, the peer can retransmit its FIN, and the host can resend the ACK to properly close the connection. It also allows any delayed segments from this connection to expire, preventing them from interfering with future connections using the same ports. If another FIN arrives before the timer expires, the host resends the ACK and resets the TIME_WAIT timer.

4. (6 points) What is the difference between flow control and congestion control in TCP? Assume the CWND is 10 MSS, while the RWND is 8 MSS, what will be the effective transmission window size?

Flow control in TCP prevents the sender from overwhelming the receiver's buffer capacity, using the receiver's advertised window (RWND) to limit how much data can be sent. Congestion control prevents the sender from overwhelming the network, adjusting the congestion window (CWND) based on perceived network congestion (e.g., via packet loss). The effective transmission window size is the minimum of CWND and RWND, so min(10 MSS, 8 MSS) = 8 MSS.

5. (6 points) TCP congestion control uses AIMD for fairness. Please think about what will happen if TCP CC uses AIAD (additive increase and additive decrease (increase/decrease by one)) or MIAD (multiplicative increase and additive decrease). Please follow the diagram plotted in the lecture to see what will happen with two flows.

TCP's AIMD (Additive Increase, Multiplicative Decrease) ensures fairness and efficiency among competing flows by slowly increasing CWND (by 1 per RTT) and halving it on congestion, allowing flows to converge to equal shares on the fairness line in the phase plot.

If using AIAD (Additive Increase, Additive Decrease), the increase and decrease vectors would be parallel (both additive, e.g., +1 and -1), so flows would maintain their relative shares without converging to fairness. A flow starting with a larger CWND would stay ahead, leading to unfair bandwidth allocation.

If using MIAD (Multiplicative Increase, Additive Decrease), the multiplicative increase would cause aggressive growth, potentially overshooting capacity, while additive decrease reduces slowly. In the phase plot, flows might oscillate or diverge from the efficiency line, leading to instability, unfairness, or frequent congestion collapses without converging properly.

6. (12 points) Assume TCP congestion control uses all the following algorithms covered in our lecture: slow start (SS), congestion avoidance (CA), fast retransmit and fast recovery, and retransmission upon timeout.

All the following assumptions are the same as those used in class unless specified (say, cwnd, ssthresh and which packet gets lost).

• MSS = 1B. Initially ssthresh at the sender is set to 4, and cwnd as 1. Assume cwnd and ssthresh are measured in segments (or in bytes as MSS = 1B). The transmission time for each segment is negligible.

Retransmission timeout (RTO) is initially set to 500 ms at the sender and is unchanged during the connection lifetime. RTT is 100ms for all transmissions.

• The connection starts to transmit data at time t = 0, and the initial sequence number starts from 1. Segment with sequence number 9 is lost once. No other segments are lost.

• The receiver acknowledges every segment (no delayed ACK) and uses TCP cumulative ACK (i.e., ACK = (largest sequence received in order at the receiver + 1) ). Besides, the sender always has data available for transmission.

• When ssthresh equals to cwnd, we use slow start, not congestion avoidance.

• Right after fast retransmit/fast recovery phase, if ssthresh equals to cwnd, use the slow start algorithm.

• The receiver will buffer out of order packets (like selective repeat).

When does the sender receive the ACK for the segment with the sequence number 15? Which algorithms will be used upon receiving ACK for the segments with the sequence number 15 and 16? Show your diagram. (hints: please follow the example diagram in class to show congestion control step-by-step.)

The sender receives the ACK for the segment with sequence number 15 (i.e., ACK 16, which acknowledges up to segment 15) at t = 500 ms.

Upon receiving the ACK for segment 15 (ACK 16), fast recovery is used (to exit the phase and deflate CWND to ssthresh). Upon receiving the ACK for segment 16 (ACK 17), slow start is used.

Below is a step-by-step diagram of the congestion control process, following the class example style. Each "round" corresponds to an RTT of 100 ms. CWND and ssthresh are in segments. The diagram shows CWND evolution, segments sent, and key events.

| Time (ms) | Round | Mode at Start | CWND Start | ssthresh | Segments Sent | Events / ACKs Received | CWND Update |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0-100 | 1 | Slow Start | 1 | 4 | 1 (seq 1) | ACK 2 | CWND = 2 |
| 100-200 | 2 | Slow Start | 2 | 4 | 2 (seq 2-3) | ACK 3, ACK 4 | CWND = 4 |
| 200-300 | 3 | Slow Start (CWND == ssthresh) | 4 | 4 | 4 (seq 4-7) | ACK 5-8 | CWND = 8 |
| 300-400 | 4 | Congestion Avoidance (CWND > ssthresh) | 8 | 4 | 8 (seq 8-15) | ACK 9 (new, for 8; +1/8 to CWND ≈8.125), then 6 dup ACK 9 (for 10-15). At 3rd dup: fast retransmit seq 9, ssthresh=4, CWND=4+3=7. Then 3 more dups: CWND+=1 each → CWND=10. Send new seq 16-18 during recovery. | CWND=10 (inflated in recovery) |
| 400-500 | 5 | Fast Recovery | 10 | 4 | Retr 9, 16-18 | ACK 16 (new, acks 9-15; exit fast recovery, set CWND=4), then ACK 17-19 (new, for 16-18; since CWND==ssthresh post-recovery, use Slow Start: CWND+=1 per ACK → CWND=7 after these) | CWND=7 (after all ACKs processed) |